Project Requirements

Project Title:
Adjustable Motor Mount for a NEMA 17 Stepper Motor

Project Goal:
Design a simple and adjustable motor mount for a NEMA 17 stepper motor. The mount should hold the motor securely and allow linear adjustment of the motor position for alignment or belt tensioning.

Main Requirements:

| Requirement ID | Requirement | Target / Value | Verification Method |
| --- | --- | --- | --- |
| REQ-01 | The motor plate shall interface with the simplified NEMA 17-style CAD model. | The 40 x 40 mm motor body is only a simplified CAD representation, not a verified commercial NEMA 17 envelope. The mounting interface uses four M3 holes on a 31 x 31 mm pattern. Final fit with a specific real motor requires verification. | Check the CAD assembly and mounting-hole alignment. Before manufacture, compare the drawing against the selected motor's datasheet and perform a physical fit check. |
| REQ-02 | The motor plate shall retain the current compact geometry. | 70 x 70 x 5 mm nominal. | Measure the CAD model or inspect the released drawing. |
| REQ-03 | The base plate shall retain the current mounting envelope. | 100 x 90 x 6 mm nominal. | Measure the CAD model or inspect the released drawing. |
| REQ-04 | Each adjustment slot shall match the final CAD geometry. | 6 mm width, 18 mm between end-arc centers, and 24 mm overall length. | Measure the slots in the CAD model or released drawing. |
| REQ-05 | The motor position shall be linearly adjustable along one axis. | At least 18 mm of usable linear adjustment. | Measure the motor displacement between the two valid extreme positions in CAD or on a prototype. |
| REQ-06 | Components shall remain clear throughout the full adjustment range. | No interference between the motor, plates, shaft, slots, or fasteners across the complete 18 mm travel. | Run a SolidWorks interference check at both travel limits and inspect intermediate positions. |
| REQ-07 | The adjustment joint shall retain its selected position during representative bench operation. | No visible slip under a 30 N lateral static load. The 30 N load is a provisional portfolio design assumption, not a verified operating or safety rating. | Apply a 30 N lateral load to the mounted motor and compare its position before and after the test. |
| REQ-08 | Structural stresses shall remain below the selected material's yield strength under the assumed load. | Calculated factor of safety >= 2 under the provisional 30 N lateral static load. The factor-of-safety target is a provisional portfolio design assumption and must be reassessed when actual operating loads are known. | Perform documented calculations using stated material properties, geometry, boundary conditions, and load direction. |
| REQ-09 | The motor and adjustable joint shall use standard fasteners. | Four M3 motor fasteners and standard M5 adjustment fasteners consistent with the current CAD components. | Inspect the CAD assembly, drawings, and BOM; verify clearance and thread engagement. |
| REQ-10 | The mount shall provide attachment points for connection to an external base. | At least four accessible base-mounting locations sized for the selected standard fasteners. | Inspect the base-plate drawing and perform a CAD or physical fit check with the intended support structure. |
| REQ-11 | The custom plates shall be practical to manufacture using basic workshop processes. | Flat-plate construction using conventional cutting, drilling, and slot-machining operations; specialized processes must be documented if required. | Conduct a manufacturing review of the final part drawings. |
| REQ-12 | Installation and adjustment shall be possible using common hand tools. | All required fasteners accessible with standard hex keys or wrenches; motor position adjustable without removing the motor from its plate. | Conduct an assembly and adjustment demonstration. |
| REQ-13 | The released design shall have a minimum manufacturing and assembly drawing package. | Required drawing-package content: dimensioned drawings for the motor plate and base plate, plus an assembly drawing showing units, materials, general tolerances, fasteners, and adjustment direction. | Review the completed drawing package against a release checklist. |

Selected Motor:
Simplified NEMA 17-style CAD representation

Initial Motor Data:

- Motor body size: 40 x 40 mm simplified CAD representation; not a verified commercial NEMA 17 envelope

- Mounting hole pattern: 31 x 31 mm

- Mounting holes: M3

- Shaft diameter: approximately 5 mm

Initial Design Dimensions:

- Motor Plate: 70 x 70 x 5 mm

- Base Plate: 100 x 90 x 6 mm

- Adjustment Slots: 6 mm wide, 18 mm between end-arc centers, and 24 mm overall length

- Standard fasteners will be used where possible

Planned Deliverables:

- SolidWorks part models

- Full assembly

- Engineering drawings

- Exploded view

- Bill of Materials (BOM)

- Basic GD&T

- Basic design calculations

- Final project documentation